From DFT to MLIP: A Lightweight Materials Modeling Pipeline for Semiconductor Metrology

Why Modeling Matters in Semiconductor Metrology

Modern semiconductor devices are engineered at the scale of atoms. As device dimensions shrink and new materials enter fabrication lines, understanding how atomic structures influence measurable signals becomes increasingly important. Metrology tools—such as electron microscopy, X-ray techniques, and optical inspection—can capture extremely detailed information about materials. However, interpreting those signals often requires a model of the underlying atomic structure, as experimental observations alone will not reveal the faulty mechanism in materials.

Materials modeling helps bridge this gap. By simulating how atoms arrange themselves and how defects form, modeling can provide a reference for interpreting measurements. For semiconductor companies working with advanced materials like gallium nitride (GaN), modeling can help answer practical questions:

- How do atomic defects affect measurable signals?

- What structures are stable under realistic conditions?

- What kinds of defects or interface quality exist in the material?

Traditionally, answering these questions requires density functional theory (DFT)—a quantum mechanical method widely used to compute energies and forces in materials. While DFT is powerful, it is also computationally expensive, especially when studying large structures or complex defects.

This is where machine learning is beginning to play a role.

The Challenge: Accuracy vs. Scale

DFT calculations can describe materials with high fidelity, but they scale poorly with system size. A simulation containing a few dozen atoms might be manageable, but semiconductor defects, interfaces, and extended structures can easily require hundreds or thousands of atoms.

In practice, this creates a trade-off:

- DFT: accurate but computationally heavy

- Large-scale simulations: necessary for real defects but difficult with pure quantum methods

Machine-learned interatomic potentials (MLIPs) aim to bridge this gap. Instead of computing quantum interactions directly each time, an ML model learns the relationship between atomic configurations and the forces or energies produced by DFT. Once trained, the model can evaluate structures much faster.

The result is a workflow where DFT provides accurate reference data, and machine learning enables scaling simulations to larger systems.

A Minimal Pipeline: From DFT to MLIP

To demonstrate this idea, I built a lightweight modeling pipeline using wurtzite gallium nitride (GaN) as a case study. GaN is widely used in power electronics, RF devices, and optoelectronics, making it a relevant material for semiconductor metrology and characterization.

The pipeline intentionally focuses on a simple but complete workflow:

Structure generation → DFT labeling → dataset extraction → MLIP training → large-scale relaxation → validation

1. Structure Generation

The process starts by generating atomic structures. These include:

- bulk GaN supercells

- simple vacancy defects

- slightly perturbed (“rattled”) structures

These variations provide the diversity needed for training a machine-learning potential.

2. DFT Labeling

Each structure is evaluated using DFT calculations. The output provides:

- total energies

- atomic forces

- relaxed atomic positions

These results form the ground truth dataset used to train the machine learning model.

3. Dataset Construction

The DFT results are converted into a format suitable for ML training (such as extxyz), where each structure is paired with its corresponding energy and force information.

Although the dataset in this demo is intentionally small, the workflow mirrors how larger datasets are constructed in industrial modeling pipelines.

4. Training a Machine-Learned Interatomic Potential

Using the generated dataset, a machine-learned interatomic potential is trained using the MACE framework. The model learns how atomic arrangements map to energies and forces.

After training, the MLIP can predict forces and energies much faster than DFT, enabling simulations on larger systems.

5. Scaling to Larger Structures

With the trained MLIP, we can simulate larger GaN systems—hundreds of atoms instead of a few dozen. This allows exploration of defect configurations or extended structures that would be expensive to study with DFT alone.

The MLIP can relax these structures and provide approximate energies and atomic positions.

6. Validation with DFT Spot Checks

To ensure the ML model remains physically reasonable, selected configurations are re-evaluated with DFT. These spot checks compare ML predictions with quantum calculations using simple validation metrics such as:

- energy differences per atom

- force errors on defect atoms

This step provides a basic quality gate for the model.

Connecting Structure to Metrology

The ultimate goal of modeling in semiconductor workflows is not simply to compute energies—it is to connect atomic structure to observable signals.

In this project, relaxed GaN structures can be used to generate synthetic microscopy images using simulation tools such as abTEM, which can produce STEM-like images from atomic models. Even simple simulations can help illustrate how defects or structural changes may influence the signals measured by experimental instruments.

By combining:

- atomistic modeling

- machine-learned potentials

- simulated observables

we can begin to create a loop between structure prediction and measurement interpretation.

Why This Matters

While the demonstration pipeline is intentionally lightweight, it illustrates several ideas that are becoming increasingly important in semiconductor research:

- Hybrid modeling workflows
Combining physics-based simulations with machine learning enables scaling beyond traditional limits.

- Data-driven materials modeling
Small datasets can seed models that expand exploration of atomic configurations.

- Integration with experimental observables
Simulated structures can help interpret microscopy and metrology signals.

- Reproducible computational pipelines
Packaging modeling workflows into structured pipelines makes them easier to reproduce and extend.

Looking Ahead

Machine learning will not replace physics-based modeling, but it is rapidly becoming a complementary tool. In semiconductor materials research, ML-assisted simulations may help accelerate tasks such as:

- defect exploration

- interface modeling

- interpretation of microscopy data

- screening of material structures

Even simple pipelines—like the GaN example described here—demonstrate how quantum simulations, machine learning, and experimental interpretation can begin to work together.

As device complexity continues to grow, combining modeling and metrology may become an increasingly important part of semiconductor engineering workflows.

This project demonstrates a minimal, reproducible example of such a workflow: using DFT data to train a machine-learned potential and exploring larger GaN structures while maintaining a connection to experimental observables.