From DFT to MLIP: A Lightweight Materials Modelling Pipeline for Semiconductor Metrology

Why Modelling Matters in Semiconductor Metrology

Modern semiconductor devices are engineered at the scale of atoms. As device dimensions shrink and new materials enter fabrication lines, understanding how atomic structures influence measurable signals becomes increasingly important.

Metrology tools—such as electron microscopy, X-ray techniques, and optical inspection—can capture extremely detailed information about materials. However, interpreting those signals is not always straightforward. Experimental observations alone often cannot reveal the underlying defect mechanisms responsible for device performance issues.

Materials modelling helps bridge this gap. By simulating how atoms arrange themselves and how defects form, modelling provides a reference framework for interpreting measurements.

For semiconductor materials such as gallium nitride (GaN), modelling can help answer practical questions:

- How do atomic defects affect properties?

- What structures remain stable under realistic conditions?

- What types of defects or interface quality might exist in the material?

Traditionally, answering these questions requires density functional theory (DFT)—a quantum mechanical method widely used to compute energies and forces in materials. While DFT is extremely powerful, it is also computationally expensive, particularly when studying large structures or complex defects.

This is where machine learning is beginning to play an important role.

The Challenge: Accuracy vs. Scale

DFT calculations can describe materials with high fidelity, but they scale poorly with system size. A simulation containing a few dozen atoms may be manageable, but realistic semiconductor defects, interfaces, and extended structures can easily require hundreds or thousands of atoms.

This creates a familiar trade-off:

- DFT: accurate but computationally expensive

- Large-scale simulations: necessary for realistic defects but difficult using quantum methods alone

Machine-learned interatomic potentials (MLIPs) aim to bridge this gap. Instead of solving the full quantum problem every time, an ML model learns the relationship between atomic configurations and the energies and forces produced by DFT.

Once trained, the model can evaluate new atomic structures orders of magnitude faster than DFT.

In practice, this leads to a hybrid workflow where:

- DFT provides accurate reference data, and

- Machine learning enables simulations at much larger scales.

A Minimal Pipeline: From DFT to MLIP

To demonstrate this idea, I built a lightweight modelling pipeline using wurtzite gallium nitride (GaN) as a case study. GaN is widely used in power electronics, RF devices, and optoelectronics, making it a relevant material for semiconductor applications.

The pipeline intentionally focuses on a simple but complete workflow:

Structure generation → DFT labeling → dataset extraction →

MLIP training → large-scale relaxation → validation

Even though the demonstration dataset is small, the workflow can be easily extended to the larger industrial modelling pipelines.

Essential Commands That Drive the Workflow

Below are a few representative commands that illustrate how the pipeline works in practice. These commands highlight the key parameters typically tuned during modeling workflows.

DFT Labeling

The first step generates reference energies and forces using DFT.

```bash

MLIP_GPAW_GPU_ECUT=350 python dft/scripts/tier_b_calculations.py --gpu --calc-type single_point --structure-ids GaN_bulk_sc_2x2x2 --max-structures 1 --maxiter 25 --conv-energy 1e-3 --conv-density 1e-2 --conv-eigenstates 1e-4 --mag-config none

```

Important parameters include:

- plane-wave cutoff energy

- SCF convergence thresholds

- maximum iteration count

These parameters control the stability and accuracy of the DFT calculation.

Dataset Extraction

Once DFT calculations finish, the results are collected and converted into a training dataset.

```bash

python dft/scripts/extract_dft_data.py --max-atoms 300

```

This step gathers completed calculations and writes them into training formats such as extxyz, which store atomic structures together with energies and forces.

Training the ML Potential

Next, a machine-learned interatomic potential is trained using the MACE framework. MACE is a modern neural network architecture designed specifically for atomistic simulations. Instead of treating atoms independently, it models how atoms interact with their neighbors by passing information through a graph representing the atomic structure. Each atom receives messages from nearby atoms, allowing the model to learn how local environments determine forces and energies.

```bash

python mlip/scripts/train_mlip.py --max-epochs 80 --patience 10 --eval-interval 20 --energy-weight 10.0 --forces-weight 10.0

```

The model learns how atomic structures map to forces and energies.

Once trained, the ML potential can evaluate new configurations far faster than DFT.

Validation Gates

Before applying the model to larger systems, quick validation checks are performed.

Energy comparison example:

```bash

python analysis/scripts/energy_gate.py --model path/to/model.model --dft-json dft/results/tier_b_results.json --device cuda --threshold 0.01 --case GaN_bulk_sc_4x4x4_relaxed:dft/structures/GaN_bulk_sc_4x4x4_relaxed.cif

```

These simple “validation gates” ensure that the ML model remains reasonably consistent with the original DFT reference data.

Scaling Simulations to Larger Structures

With the trained ML potential, simulations can be applied to larger GaN supercells—containing hundreds of atoms rather than a few dozen.

The MLIP can relax these structures and estimate their energies and forces quickly. This allows exploration of defect configurations and structural variations that would otherwise be expensive to compute using DFT alone.

To maintain physical reliability, selected configurations are occasionally re-evaluated with DFT as spot checks.

Connecting Structure to Metrology

The ultimate goal of modeling in semiconductor workflows is not simply to compute energies—it is to connect atomic structure to measurable signals.

In this project, relaxed GaN structures can be used to generate synthetic microscopy images using simulation tools such as abTEM, which can produce STEM-like images directly from atomic models. One can also calculate optical properties for the large supercells imposed on defects and stress/strain. The atomic structure can be validated by atomic resolution microscopy images.

By combining:

- atomistic modeling

- machine-learned potentials

- simulated observables

We can begin to build a bridge between structure prediction and measurement interpretation.

Why This Matters

Although this demonstration pipeline is intentionally lightweight, it illustrates several ideas that are becoming increasingly important in semiconductor research:

Hybrid modelling workflows
Combining physics-based simulations with machine learning enables scaling beyond traditional computational limits.

Data-driven materials modelling
Even relatively small datasets can seed machine-learned models that expand exploration of atomic configurations.

Integration with experimental observables
Simulated structures can help interpret signals from microscopy and metrology tools.

Reproducible computational pipelines
Packaging modelling workflows into structured pipelines makes them easier to reproduce and extend.

Looking Ahead

Machine learning will not replace physics-based modelling, but it is rapidly becoming an important complementary tool. In semiconductor materials research, ML-assisted simulations may help accelerate tasks such as:

- defect exploration

- interface modelling

- interpretation of microscopy data

- screening of new material structures

As semiconductor devices continue to scale and material complexity increases, integrating modeling with metrology may become an essential part of future engineering workflows.

This project demonstrates a minimal, reproducible example of such a workflow: using DFT data to train a machine-learned potential and exploring larger GaN structures while maintaining a connection to experimental observables.